WT Exp-6

- Aim :-

Interface the Arduino, connect the components together and run the project.

- Code :-

#include <SPI.h>

#include <MFRC522.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#define SS_PIN 10

#define RST_PIN 9

#define BUZZER 2 // Buzzer pin

#define SERVO_PIN 6 // Servo pin

MFRC522 mfrc522(SS_PIN, RST_PIN);  // RFID instance

Servo myServo;                     // Servo instance

LiquidCrystal_I2C lcd(0x27, 16, 2); // LCD instance with I2C address 0x27

void setup() {

Serial.begin(9600);

SPI.begin();

mfrc522.PCD_Init();

myServo.attach(SERVO_PIN);

myServo.write(0); // Ensure servo starts at 0°

pinMode(BUZZER, OUTPUT);

noTone(BUZZER);

lcd.init();

lcd.clear();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("  Put card on   ");

lcd.setCursor(0, 1);

lcd.print("     reader     ");

}

void loop() {

if (!mfrc522.PICC_IsNewCardPresent()) {

return;

}

if (!mfrc522.PICC_ReadCardSerial()) {

return;

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("UID tag:");

String content = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

lcd.print(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " ");

lcd.print(mfrc522.uid.uidByte[i], HEX);

content.concat(String(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " "));

content.concat(String(mfrc522.uid.uidByte[i], HEX));

}

content.toUpperCase();

lcd.setCursor(0, 1);

if (content.substring(1) == "53 37 5A E4") { // Replace with your card's UID

lcd.print("Access Granted");

tone(BUZZER, 500);

delay(300);

noTone(BUZZER);

myServo.write(90);  // Move servo to 90° (open position)

delay(3000);        // Hold open for 3 seconds

myServo.write(0);   // Move servo back to 0° (closed position)

delay(1000);        // Small delay before resuming loop

} else {

lcd.print("Access Denied");

tone(BUZZER, 300);

delay(2000);

noTone(BUZZER);

}

delay(2000);

}

- Practical :-

Connecting the Arduino with all of its components :

### RFID Reader to Arduino (SPI Protocol)

| RFID Reader Pin | Arduino Pin | Description |
| --- | --- | --- |
| SDA | 10 | Slave Select (SS) |
| SCK | 13 | Serial Clock (SCK) |
| MOSI | 11 | Master Out Slave In (MOSI) |
| MISO | 12 | Master In Slave Out (MISO) |
| GND | GND | Ground |
| 3.3V | 3.3V | Power Supply |

### LCD Display to Arduino (I2C Protocol)

| LCD Display Pin | Arduino Pin | Description |
| --- | --- | --- |
| SCL | A5 | Serial Clock Line (SCL) |
| SDA | A4 | Serial Data Line (SDA) |
| VCC | 5V | Power Supply |
| GND | GND | Ground |

### Servo Motor to Arduino

| Servo Motor Pin | Arduino Pin | Description |
| --- | --- | --- |
| Control Pin | D7 | Signal Control |
| Power | 5V | Power Supply |
| Ground | GND | Ground |

### Buzzer to Arduino

| Buzzer Pin | Arduino Pin | Description |
| --- | --- | --- |
| + (Positive) | D2 | Signal Input |
| - (Negative) | GND | Ground |

- Circuit Diagram :-